APCS

Adaptive Path & Collision-avoidance System

Team briefing document — Smart India Hackathon 2026 · Problem statement SIH26037

Purpose of this document: a shared reference for the whole team to read, discuss, and align on before the pitch/demo. It covers what APCS is, how it's built, what's real vs. simulated in the prototype, and how we'll run the live demo. Update the status sections as the build progresses.

# 1. The Problem

Large stretches of Indian roads — especially in hill and mountain regions — carry serious, time-varying hazards: landslides, flash floods, avalanches, and glacial lake outburst floods (GLOF), on top of everyday on-road hazards like potholes and stray animals. Drivers plan routes without any of this risk information baked in, and existing navigation apps treat every road as equally safe.

# 2. Our Solution — What APCS Does

APCS is a lightweight route-planning system that ingests hazard data along a driving corridor, scores risk per road segment, fuses it into a stable score, feeds that into route computation as a cost penalty, and generates a driver-facing alert only when a real risk state actually changes. In one line:

It plans a normal route like any map app — but reroutes around real, confirmed hazards, and explains why in plain language, without wasting compute on every noisy sensor blip.

# 3. System Architecture — Six Layers

Data flows through six strictly ordered layers. Only the risk-fusion layer (L4) and the LLM agent (L6) touch a shared time-series state store, which is what lets the system reason about trends instead of overreacting to a single reading.

| Layer | Responsibility | Key design choice |
| --- | --- | --- |
| L1 | Data ingestion | Scheduled pollers per source, aggressive caching, respects free-tier rate limits |
| L2 | Geospatial feature store | Buffers the route into a 10–20 km corridor, tiles it with H3, writes RoadSegmentRisk rows |
| L3 | Hazard prediction | One GBM model per hazard (landslide, flood, avalanche); CNN reserved for glacier/GLOF imagery |
| L4 | Risk fusion and state | Combines probability, severity, exposure, confidence; applies hysteresis |
| L5 | Routing engine | OSRM (Docker + OSM data); risk becomes an edge-weight penalty — never computed by the LLM |
| L6 | LLM agent, gated | Small local model (Ollama), invoked only past threshold, output is structured JSON only |

## System flow, tick by tick

- GPS update arrives → L1 supplies fresh weather/terrain/satellite data for the corridor (scheduled, cached).

- L2 buffers the route into a 10–20 km corridor and tiles it with H3.

- L3 scores each segment: GBM models for landslide/flood/avalanche; CNN (stubbed) for glacier/GLOF.

- L4 fuses scores and applies hysteresis — this is the first gate. On a normal drive, the loop stops here: cheap, plain code, no ML/LLM cost.

- If fused risk crosses threshold, L5 (OSRM) recomputes the route, treating risk as a cost penalty. Routing is always decided by the graph-search engine, never by the LLM.

- Only on a confirmed state transition (threshold crossed AND hysteresis holds) does the gated LLM agent (L6) run, call up to three tools, and return structured JSON only.

- That JSON becomes the driver-facing alert or reroute explanation.

- If live data drops, the system falls back to last-known risk + static terrain risk — the driver is never shown “no warning” purely because a feed dropped.

Presentation point to lead with: on the large majority of ticks, the flow never leaves the L4 gate. That's what keeps the system lightweight in practice, not just in theory.

# 4. Tech Stack (prototype)

- Frontend: React + Leaflet (OpenStreetMap tiles) — a web app, so it runs on any laptop for the live demo.

- Backend: Python (FastAPI).

- Routing: OSRM, self-hosted via Docker, over OpenStreetMap data for the demo region.

- Hazard models: XGBoost / LightGBM, one per hazard (landslide, flood, avalanche).

- Geospatial tiling: H3 hexagonal indexing.

- LLM agent: small local model via Ollama (3–8B, quantized), schema-constrained JSON output.

- Everything orchestrated with docker-compose for a one-command demo start.

# 5. Data Sources — What's Live vs. Simulated

Be upfront about this with judges — it's a design choice, not a gap. Real-time satellite access for GLOF-relevant imagery isn't realistically available on a free tier, so that one path is stubbed and clearly labelled as such.

| Hazard input | Source used in prototype | Status |
| --- | --- | --- |
| Weather (rain, temp, wind) | Open-Meteo (free, no API key) | Live |
| Terrain / slope / aspect | Open-Elevation / OpenTopography DEM | Live |
| Glacier / GLOF imagery | Static sample imagery, CNN interface built but not live | Stubbed (flagged openly) |
| Road network | OpenStreetMap via self-hosted OSRM | Live |

Hazard model training note: since real labelled hazard datasets aren't available for the demo region, the GBM models are trained on a small rule-based/synthetic label set built from the real weather + DEM features (e.g. steep slope + heavy rainfall → higher landslide probability). The models themselves are real and trained — only the ground-truth labels are synthetic. Say this plainly if asked.

# 6. Design Rules We Do Not Break

- Routing decisions are made only by OSRM / graph search — never by the LLM.

- The LLM never computes risk, paths, or collision-avoidance manoeuvres.

- LLM output is JSON only, schema-constrained — no UI logic depends on parsing prose.

- Data scope is bounded to the route corridor (10–20 km), not global.

- The LLM is gated behind two cheap checks (threshold + hysteresis) that run in plain code on every tick.

# 7. Live Demo Script

| Step | What we do | What it proves |
| --- | --- | --- |
| 1 | Plan a route through the demo corridor; map loads with no hazard overlay | Baseline routing works end-to-end |
| 2 | Let the drive run normally — no hazard nearby | System stays cheap: it stops at the L4 gate, no ML/LLM cost incurred |
| 3 | Trigger a hazard scenario (e.g. inject heavy rain + steep segment) | Risk score rises; a single noisy spike alone does NOT reroute (hysteresis) |
| 4 | Sustained risk rise crosses threshold | State transition confirmed → route recomputes automatically, ETA delta shown |
| 5 | Gated LLM agent fires exactly once, for this transition | Structured JSON alert renders as a clean driver-facing card |
| 6 | Toggle "kill weather feed" in the debug panel | Fallback: last-known state + static terrain risk keeps a warning visible — never silent |

# 8. Explicitly Out of Scope (say this if asked)

- National-scale / multi-region deployment.

- Certified emergency-broadcast integration.

- Full autonomous (Level 4/5) vehicle control — this is decision support, not vehicle actuation.

- Heavier deep-learning hazard models (Physics-GNN for flash flood, ConvLSTM+ResNet for avalanche) — deferred in favour of GBM, which needs one serving stack and works with small datasets.

- Native Android client for this round — web app first.

# 9. Anticipated Judge Questions

- “Why not just use a bigger/frontier LLM for everything?” — Because it's gated: the LLM only runs on confirmed transitions, and it never decides risk or routes. That keeps latency and cost near zero on normal drives, and keeps safety-critical decisions in deterministic code, not model output.

- “Is this real data or a simulation?” — Weather and terrain are live, free-tier APIs. Glacier/GLOF imagery is stubbed and we say so. Hazard model labels are synthetic/rule-based since real labelled datasets for this region don't exist; the models themselves are genuinely trained, not hardcoded.

- “What happens if the network/API drops mid-drive?” — Falls back to last-known risk plus static terrain risk. The driver is never shown a false all-clear.

- “Could this scale beyond the demo region?” — Architecture supports it (per-hazard models are independently trainable, H3 tiling is region-agnostic); national-scale deployment is explicitly out of scope for this prototype.

# 10. Team Role Mapping

From the architecture document — adjust names/assignments as needed so every teammate can speak to their piece on stage.

| Area | Covers |
| --- | --- |
| AI/ML members | L1–L4: ingestion, feature store, hazard GBM/CNN models, risk fusion & hysteresis, plus L6 LLM agent and tool schema |
| App/CS member (docs owner) | L5 routing integration on the client side, map/alert UI, notification handling, offline fallback UX, and the SRS |
| Remaining members | Fill in — assign to a phase (1–6) or a layer above so every teammate owns a piece they can speak to on stage |

# 11. Build Status

Fill in as the prototype comes together:

- ☐ Phase 1 — Routing core (OSRM + corridor + H3 tiling + map UI)

- ☐ Phase 2 — One hazard end to end (landslide via GBM)

- ☐ Phase 3 — Fusion + hysteresis

- ☐ Phase 4 — Routing integration (risk as edge weight, reroute + ETA delta)

- ☐ Phase 5 — Gated LLM agent (Ollama + 3 tools + JSON schema)

- ☐ Phase 6 — Remaining hazards (flood, avalanche, glacier stub) + fallback